DIGITAL COLLECTION BASICS: QUICK REFERENCE GUIDE

Washington State Archives | Sno-Isle Library Presentation Maggie Cogswell, Digital Projects Archivist

BEFORE YOU START: KEY DECISIONS

Should You Digitize?

✓ Items are deteriorating or at risk

✓ Obsolete formats (VHS, cassettes, etc.)

✓ Frequent access needed

✓ You can commit to 3-2-1 backup + 5-year migration

✗ Items are stable in good storage

✗ You cannot maintain backups long-term

For help assessing condition & obsolete formats: See Planning Worksheet page 15 of the Personal Digital Archiving Planning Worksheet.

SCANNING SETTINGS AT-A-GLANCE

Resolution (PPI/DPI):

- Documents: 300-400 PPI

- Standard photos: 600 PPI

- Photos for enlargement: 600-1200 PPI

- Slides/negatives: 1200+ PPI

Color Mode:

- Color (24-bit): Color photos, maps, anything where color matters

- Grayscale (8-bit): Black & white photos, manuscripts

- Bitonal (1-bit): Typed documents ONLY (never photos)

File Formats:

- Master files: TIFF (uncompressed)

- Access copies: JPEG (high quality) or PDF

- Multi-page documents: Multi-page PDF

Golden Rule: If scanning only once, scan at highest quality you can manage!

FILE NAMING TEMPLATE

Pattern: YYYY-MM-DD_description_###.extension

Example: 1965-07_mom_wedding_001.tif

Rules:

- Use underscores (_) or hyphens (-), NOT spaces

- Use leading zeros (001, 002, 003)

- Avoid special characters: \ / : * ? " < > | [ ] & $ , .

- Keep under 50 characters

- Be consistent!

FOLDER STRUCTURE BASICS

My_Digital_Collections

Photographs

Masters_TIFF  [organize by date/family/subject]

Access_JPEG  [same organization as masters]

Documents

Masters

Access

Audio_Video

Masters

Access

Key Principle: Keep masters separate from access copies!

THE 3-2-1 BACKUP RULE (NON-NEGOTIABLE!)

3 = Three copies of every important file 2 = Two different types of media 1 = One copy stored off-site

Example Setup:

- Working copy: Computer hard drive

- Backup #1: External hard drive at home

- Backup #2: Cloud storage OR external drive at relative's house

Set Reminders NOW:

- Annual check: __________ (1 year from today)

- 5-year migration: __________ (5 years from today)

METADATA: CAPTURE WHILE YOU REMEMBER

Minimum fields to record:

- Filename

- Original date (approximate okay)

- Description

- People shown (if known)

Best tool: Simple spreadsheet (Excel, Google Sheets)

Warning: Social media strips embedded metadata - keep a separate spreadsheet!

WHEN TO SEEK PROFESSIONAL HELP

Consider professional services for:

- Large quantities (50+ tapes/films)

- Obsolete formats (8mm film, reel-to-reel, Betamax)

- Fragile or damaged materials

- Materials that resist unfolding/flattening

- Active mold (health hazard!)

- Items in POOR condition (see Condition Guide, page 15 of the Personal Digital Archiving Planning Worksheet)

Typical costs:

- VHS transfer: $15-30 per tape

- Audio cassettes: $10-20 per tape

- 8mm film: $0.50-3 per foot

- Slides/negatives: $0.25-1 per image

WHAT NOT TO DO

Don't clean originals before scanning - Even light dusting can cause damage

Don't force items to lay flat - Respect bindings and fragile materials

Don't use:

- PVC plastics (releases harmful gases)

- Magnetic photo albums (damages photos)

- Rubber bands or paper clips on originals

- Regular cardboard boxes for storage

Don't store in:

- Attics (too hot)

- Basements (too damp)

- Garages (temperature extremes)

PHYSICAL STORAGE BASICS

Good Environment:

- Cool (70°F or below)

- Dry (30-50% relative humidity)

- Dark (light causes fading)

- Stable (avoid fluctuations)

Best Location: Interior closet in central part of home

Proper Housing:

- Acid-free AND lignin-free boxes/folders

- PAT-tested materials for photos

- Polyester, polypropylene, or polyethylene sleeves

- Items stored 4+ inches off floor

FREE ONLINE RESOURCES

Library of Congress Personal Digital Archiving www.digitalpreservation.gov/personalarchiving

FADGI Digitization Guidelines www.digitizationguidelines.gov

National Archives Family Archives www.archives.gov/preservation/family-archives/digitizing

Getty "Preventing Digital Decay" blogs.getty.edu/iris/preventing-digital-decay

Washington State Digital Archives www.digitalarchives.wa.gov

ARCHIVAL SUPPLY VENDORS

- Hollinger Metal Edge: hollingermetaledge.com

- Gaylord Archival: gaylordarchival.com

- University Products: universityproducts.com

- Light Impressions: lightimpressionsdirect.com

REMEMBER:

✓ Start small (20-50 items)

✓ Complete one project fully before expanding

✓ Physical preservation is a valid choice

✓ If you digitize, commit to doing it right

✓ Back up BEFORE disaster strikes

✓ Set those calendar reminders NOW

Questions? Contact: Maggie Cogswell Washington State Archives, Maggie.cogswell@sos.wa.gov, www.digitalarchives.wa.gov